BLK18EV-2045-38X38-B-V1_01 Interface Description

P4 Infrared

P2  Ethernet

P7 Infrared interface

P6 Alarm Audio

P1 power

P5 IR-CUT

Detailed Definition

| Label | Socket | Specific No | Interface Description | Function |
| --- | --- | --- | --- | --- |
| P1 | J5 | 1 | NC | Undefined |
|  |  | 2 | NC | Undefined |
|  |  | 3 | GND | GND |
|  |  | 4 | +12V | 12V DC input |
| P2 | J2 | 1 | ETHTR- | Ethernet signal |
|  |  | 2 | ETHTR+ | Ethernet signal |
|  |  | 3 | LED/PHY_AD3 | LED |
|  |  | 4 | ETHTX- | Ethernet signal |
|  |  | 5 | ETHTX+ | Ethernet signal |
|  |  | 6 | LED/PHY_AD0 | LED |
| P3 | J3 | 1 | UART0_RXD |  |
|  |  | 2 | UART0_TXD |  |
|  |  | 3 | GND |  |
| P4 | J4 | 1 | Infrared input | Infrared input |
|  |  | 2 | GND | GND |
|  |  | 3 | +5V | 5V DC input |
| P5 | J6 | 1 | IRCUT_A | IRCUT output |
|  |  | 2 | IRCUT_B | IRCUT output |
| P6 | J8 | 1 | AC_OUT | AC_OUT |
|  |  | 2 | MIC_P | MIC_P |
|  |  | 3 | AGND | Analog ground |
|  |  | 4 | ALARM_IN | ALARM_IN |
|  |  | 5 | GND | GND |

| P7 | J19 | 1 | SD_CLK | SD_CLK |
| --- | --- | --- | --- | --- |
|  |  | 2 | SD_CMD | SD_CMD |
|  |  | 3 | SD_DATA0 | SD_DATA0 |
|  |  | 4 | SD_DATA1 | SD_DATA1 |
|  |  | 5 | SD_DATA2 | SD_DATA2 |
|  |  | 6 | SD_DATA3 | SD_DATA3 |
|  |  | 7 | LED_CTRL | LED_CTRL |
|  |  | 8 | GND | GND |
|  |  | 9 | Key-set | Key-set |
|  |  | 10 | BAT | RTC RTC battery input |
|  |  | 11 | GND | GND |
|  |  | 12 | ALARM_OUT | ALARM_OUT |
|  |  | 13 | USB_DM | USB_DM |
|  |  | 14 | USB_DP | USB_DP |
|  |  | 15 | +5V | +5V |
|  |  | 16 | USB_CONTRL | USB_CONTRL |