J6

1  2  3

J9

1  2  3  4… 7 8

J1

4

3

2

1

J5

1

2

J24

5

4

3

2

1

| Socket | Specific No | Interface Description | Function |  |
| --- | --- | --- | --- | --- |
| J9 | 1 | LED0 | LED/PHY_AD0 | Connect the tail line of the camera module |
|  | 2 | LED1 | LED/PHY_AD1 |  |
|  | 3 | ETHTX+ | Ethernet signal+ |  |
|  | 4 | ETHTX- | Ethernet signal- |  |
|  | 5 | ETHRX+ | Ethernet signal+ |  |
|  | 6 | ETHRX- | Ethernet signal- |  |
|  | 7 | GND | GND |  |
|  | 8 | 12V | 12V DC input |  |
| J24 | 1 | AC_OUTL | AC_OUT | Audio&Alarm |
|  | 2 | AC_INL/MICP | AC_IN |  |
|  | 3 | GND | GND |  |
|  | 4 | ALARM_IN1 | ALARM_GPIO |  |
|  | 5 | ALARM_OUT | ALARM_GPIO（Reserve） |  |
| J1 | 1 | +5V WIFI | +5Voutput | WIFI/4G module |
|  | 2 | USB_DN | USB data- |  |
|  | 3 | USB_DP | USB data+ |  |
|  | 4 | GND | GND |  |
| J5 | 1 | IR_CUTA | IR-CUT A OUT | IR_CUT |
|  | 2 | IR_CUTB | IR-CUT B OUT |  |
| J6 | 1 | IR | Infrared input | Connect the camera lamp board |
|  | 2 | GND | GND |  |
|  | 3 | PWM | PWM-OUT |  |

J5

15  13 ...5  3  1

16  14 ...6  4  2

J4

1

2

3

4

5

ANT2

J2

5  4  3  2  1

J3

5  4  3  2  1

| Socket | Specific No | Interface Description | Function |  |
| --- | --- | --- | --- | --- |
| ANT2 |  | RF_TR | IPEX connector | Connect IPEX to SMA feeder |
| J4 | 1 | UART_TX | UART_TX | serial port |
|  | 2 | UART_RX | UART_RX |  |
|  | 3 | GND | GND |  |
|  | 4 | NC | NC |  |
|  | 5 | +3.3V | +3.3V out |  |
| J2 | 1 | +5V | +5V in | WIFI 2.0mm |
|  | 2 | USB_DM | USB_DM |  |
|  | 3 | USB_DP | USB_DP |  |
|  | 4 | GND | GND |  |
|  | 5 | USB_CTRL | WIFI power supply IO |  |
| J3 | 1 | +5V | +5V in | WIFI 1.25mm 5 PIN |
|  | 2 | USB_DM | USB_DM |  |
|  | 3 | USB_DP | USB_DP |  |
|  | 4 | GND | GND |  |
|  | 5 | USB_CTRL | WIFI power supply IO |  |
| J5 | 1 | NC | NC | WIFI FPC |
|  | 2 | NC | NC |  |
|  | 2 | NC | NC |  |
|  | 3 | NC | NC |  |
|  | 4 | NC | NC |  |
|  | 5 | NC | NC |  |
|  | 6 | NC | NC |  |
|  | 7 | NC | NC |  |
|  | 8 | GND | GND |  |
|  | 9 | NC | NC |  |
|  | 10 | NC | NC |  |
|  | 11 | GND | GND |  |
|  | 12 | NC | NC |  |
|  | 13 | USB_DM | USB_DM |  |
|  | 14 | USB_DP | USB_DP |  |
|  | 15 | +5V | +5V in |  |
|  | 16 | USB_CTRL | WIFI power supply IO |  |