BLK16E-0235-32X32-S-V1_01  Interface Description

P3 Power& Ethernet

P5 Infrared

P4 IR-CUT

P2 Audio1/1

| sign | Socket | Specific No | Interface Description | Function |
| --- | --- | --- | --- | --- |
| P1 | J2 | 1 | UART0_RXD |  |
|  |  | 2 | UART0_TXD |  |
|  |  | 3 | GND | GND |
| P2 | J3 | 1 | AC_OUTL | AC_OUT |
|  |  | 2 | AC_INL | MIC_P |
|  |  | 3 | GND | GND |
| P3 | J1 | 1 | LED/PHYAD0 | LED |
|  |  | 2 | LED/PHYAD1 | LED |
|  |  | 3 | ETHTX+ | Ethernet signal |
|  |  | 4 | ETHTX- | Ethernet signal |
|  |  | 5 | THTR+ | Ethernet signal |
|  |  | 6 | THTR- | Ethernet signal |
|  |  |  | GND | GND |
|  |  |  | +12V | 12V DC input |
| P4 | J5 | 1 | BOUT | IR CUT |
|  |  | 2 | AOUT | IR CUT |
| P5 | J4 | 1 | Infrared input | Infrared input |
|  |  | 2 | GND | GND |
|  |  | 3 | 3.3V | 3.3V |